CS BEST

617,261 cbft / 6,874.5sqm / Built 2006

General

| Built | October – 2006 |  |  | GT | NT |
| --- | --- | --- | --- | --- | --- |
| Flag | Singapore |  | International | 14,194.0 | 6724 |
| Port of Registry | Singapore |  | Panama canal | 14,194.0 | 12,244.0 |
| Call sign | 9V9357 |  | Suez GRT/NRT | 14,857.74 | 12,312.94 |
| IMO/Lloyds nr | 9350989 |  |  |  |  |
| Length over all [m] | 162,5 |  |  | Draft | DWAT |
| Beam[m] | 26 |  | Tropical | 9,922 | 13,874 |
| Depth [m] | 14,1 |  | Summer | 9,72 | 13,107 |
| Bow thruster | 800 KW / 1000HP |  | Winter | 9,518 | 13,107 |
| Fresh Water | 9.887 / 13,121 |  | Tropical FW | 10,089 | 13,787 |

Reefer

| Bale cbft | 617,261 |
| --- | --- |
| Area m2 | 6,874.5 |
| Hatch covers | 4 |
| Holds/Compartments | 4/15 |
| Cooling sections | 1F/AB,2AB/CD,3AB/CD,4AB/CD |
| Fcsle deck | YES |
| Trunk deck | NO |
| USDA | Sensors fitted |
| PPECB | Sensors fitted |
| Reefer plugs | 200 |
| Grating load | 6 MT Axle load with pneumatic rubber tires |
| Grating | Wood |
| Temp min/max °C | -25/+15 |
| Air circs/hour | 90 |
| Air renew/hour | 2 |
| Clear deck height | 2.30 m |

CS BEST

617,261 cbft / 6,874.5sqm / Built 2006

Reefer Compartment Capacity Breakdown

Bale capacity [cbft]

| DECK & HATCH | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| FC | 55857 |  |  |  |
| A | 30406 | 54024 | 54106 | 52499 |
| B | 18402 | 43434 | 49575 | 45468 |
| C |  | 37232 | 45863 | 37239 |
| D |  | 30533 | 33842 | 28781 |
| Total | 104665 | 165223 | 183386 | 163987 |

Grandtotal	617,261cbft

Deck area [m2]

| DECK & HATCH | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| FC | 589.7 |  |  |  |
| A | 326.3 | 558.8 | 560.6 | 541.6 |
| B | 211.8 | 493.9 | 563.1 | 516.9 |
| C |  | 444.7 | 549.3 | 444.8 |
| D |  | 350.9 | 390.2 | 331.9 |
| Total | 1127.8 | 1848.3 | 2063.2 | 1835.2 |

Grand total	6874.5 m2

Cargo space volume is including the hatch way volume

Hatch sizes [m]

| Hatch | Length | Width | Insulated units |
| --- | --- | --- | --- |
| 1 | 12.6/10.29 | 10.3/6.0 | FC/AB |
| 2 | 12.6 | 10,3 | AB/CD |
| 3 | 12.6 | 10,3 | AB/CD |
| 4 | 12.3 | 10,3 | AB/CD |

CARGO GEAR

| No | Type | SWL mton | UP mton |
| --- | --- | --- | --- |
| 1,4 | hydraulic | 40 | - |
| 2,3 | hydraulic | 8 | - |

Max single lift cap:	N/A

Max twin lift cap :	N/A

MAXIMUM CONTAINER CAPACITY

|  | 20 ft (8'6'') | 40 ft (8'6'') | 40 ft (9'6'') | Stackweight 20 ft/ 40 ft |
| --- | --- | --- | --- | --- |
| WeatherDeck | 184 | 150 | 150 | 60 / 60,90,108 |
| Hatchcovers | 140 | 70 | 70 | 40,50 / 60,108 |
| Holds | 112 | 56 | 56 | 45,25,80 / 60,35,120 |

Reefer plugs:	200

Note: Maximum Reefer container capacity is approx 171 FEU at 27 MT for loaded 40 ft. Maximum container load for particular voyage is subject to Stability, Stack weight limitations, Visibility from bridge and masters approval.