SNOW FLOWER

BASICS

| Flag | SINGAPORE |
| --- | --- |
| LLoyds no. | 1039761 |
| Class | NK |
| Int GRT/NRT | 13287 / 6314 |
| Pan GRT/NRT | / |
| Suez GRT/NRT | / |
| Built | 2026 |
| Length over all m | 158 |
| Breadth moulded m | 25.0 |
| Depth W.D. m | 14.35 |
| Sdwt mton | 14549,06 |
| Designed loaded draft, mould m | 8.95 |
| Bale cbft | 637 696 |
| Area m2 | 6483,39 |
| Bow thruster |  |
| Hatch covers | 4 |
| Holds/Compartments | 4/15 |
| Fcsle deck | NO |
| Trunk deck | NO |
| USDA | Sensors fitted |
| PPECB | Sensors fitted |
| Reefer plugs | 150 |
| Grating load | Forklift with minimum 4 pneumatic rubber tires up to 6 mt total truck+cargo |
| Grating | wood |
| Temp min/max °C | -25 / +15 |
| Air circs/hour | 90 |
| Air renew/hour | 1.5/3 times |

BALE cbft

| DECK & HATCH | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| A | 37667 | 52720 | 53237 | 43351 |
| B | 25072 | 48589 | 51743 | 43124 |
| C | 20663 | 44917 | 53868 | 43209 |
| D | -- | 33799 | 48948 | 36789 |
| Total | 83402 | 180025 | 207796 | 166473 |

Grand total	637 696  cbft

DECK AREA m2

| DECK & HATCH | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| A | 339,64 | 561,70 | 576,95 | 473,29 |
| B | 240,63 | 495,01 | 548,94 | 452,04 |
| C | 204,64 | 440,41 | 568,19 | 451,39 |
| D | -- | 298,43 | 485,82 | 346,31 |
| Total | 784,91 | 1795,55 | 2179,90 | 1723,03 |

Grand total	6483,39 m2

Free deckheight minimum 2.50 m

HATCHES (Weather deck) m

| Hatch | Length | Width | Insulated units |
| --- | --- | --- | --- |
| 1 | 7.0 | 8.0 | AB/C |
| 2 | 7.0 | 8.0 | AB/CD |
| 3 | 7.0 | 8.0 | AB/CD |
| 4 | 7.0 | 8.0 | AB/CD |

Insulated units are separate cooling sections.

SNOW FLOWER

CARGO GEAR

Cranes

Nos. 1	: Cylinder luffing type Container handling deck crane

40t / 8t SWL

(hoisting speed) 14 / 42 m/min. (at 40t / 8t mode)

(slewing radius) Max. 24 m

(slewing speed) 0.35 or 1.4 rpm

Nos. 2&3	: Cylinder luffing type Pallet handling deck crane

8t SWL x 2 set

(hoisting speed) 70 m/min.

(slewing radius) Max. 22 m

(slewing speed) 1.2 rpm

Nos. 4	: Cylinder luffing type Container handling deck crane

40t / 8t SWL

(hoisting speed) 14 / 42 m/min. (at 40t / 8t mode)

(slewing radius) Max. 26 m

(slewing speed) 0.35 or 1.4 rpm

MAXIMUM CONTAINER CAPACITY

On deck & hatch cover: 152 feu + 10 teu or 154 teu + 74 feu

In cargo hold	:  45 TEU

(total)		: 152 FEU + 55 TEU or 199 TEU + 74 FEU

Reefer plug		 :  150 plugs on deck

Speed and Daily Fuel Consumption

In good weather condition i.e. up to Beaufort Force 4 and Douglas scale 3, no swell nor adverse current, and clean bottom.

Bunker grade HFO/VLSFO (IF-380CST) ISO8217 RMG380 and MGO ISO8217 DMB.

Bunker quality shall be suitable for burning in the vessel’s machineries.

Vessel entitles to use more marine gas oil at narrow / shallow / busy water area,

canals, in and out of ports and engine starting / stopping.

Service Speed:

Ballasted cond.	Designed laden cond.		Fully laden cond.

(draft abt. 6.15m)  	(draft abt. 8.95m)		(draft abt.9.7m)

For 1st- 4th yr  	abt. 18.8 knots		abt. 18.5 knots		abt. 18.0 knots

5th- 7th yr  	abt. 18.6 knots		abt. 18.3 knots		abt. 17.7 knots

8th- 10th yr 	abt. 18.3 knots		abt. 18.0 knots		abt. 17.4 knots

M/E Consumption at sea for above 3 cases:

abt. 35.2 MT/day VLSFO

or abt. 37.0 MT/day HFO when operating open loop SOx Scrubber

A/E Consumption in port :

VLSFO			Marine Gas Oil

In port (Idle)		abt. 2.3 MT			abt. 0.2 MT

At sea (Ballasted condition without cargo)①

abt. 3.6 MT			abt. 0.2 MT

At sea (at banana keeping condition without reefer container and c.a. operation)

abt. 9.1 MT			abt. 0.2 MT

At sea (Banana keeping condition + abt. 130 FEU reefer containers on deck

without c.a. operation) abt. 13.7 MT	abt. 0.2 MT

(Remarks)

1)	Additional consumption of about 1.2Mt/day for aux. engines is required during using SOx Scrubber and consuming HFO.(example,①FOC HFO 3.6+1.2MT+MGO 0.2MT)

2)	In winter condition, additional consumption of MGO about 1.2 MT/day for aux. boiler is required.

3)	Additional consumption for aux. engines is required in case of loading reefer containers and for cooling down operation.

All details believed to be correct but not guaranteed